# Metadata & Contextual Information Audit

Lane / Driving Datasets in /shared/data

Datasets reviewed: BDD100K, CULane, CurveLanes, TUSimple
Prepared for: Road Readiness Framework
Date: 2026-06-04

# 1. Executive Summary

All four datasets are lane-detection / object-detection driving datasets, not pavement-condition datasets. The purpose of this audit was to determine whether any of them carry contextual or pavement-specific information — time of capture, weather, scene / road type, lane-marking type, pavement material, marking age, or verified maintenance condition.

Key finding: None of the four datasets contain pavement material type, marking age/wear, or verified maintenance condition labels. Only BDD100K carries rich contextual metadata (weather, scene, time, GPS/IMU telemetry, and lane-marking type/color/style). CULane provides coarse scene "difficulty" categories only. CurveLanes and TUSimple provide lane geometry with essentially no contextual metadata.

# 2. Attribute Availability Matrix

| Attribute | BDD100K | CULane | CurveLanes | TUSimple |
| --- | --- | --- | --- | --- |
| Time of day | Yes – explicit label | Partial – night category only | No | Partial – via timestamp |
| Capture time / date | Yes – epoch + GPS ts | No | No | Partial – folder names |
| Weather | Yes – explicit label | No | No | No |
| Scene / road type | Yes – explicit label | Partial – implied | No | Highway only (fixed) |
| Lane marking type / color | Yes – white/yellow, single/double, solid/dashed | No – geometry only | No – geometry only | No – geometry only |
| GPS location | Yes – lat/long/altitude | No | No | No |
| Vehicle speed / IMU | Yes – speed, accel, gyro | No | No | No |
| Pavement material type | No | No | No | No |
| Marking age / wear | No | No | No | No |
| Verified maintenance condition | No | No | No | No |

# 3. BDD100K — Richest Metadata

Located at /shared/data/bdd100k/. Two annotation sources are relevant. Statistics below were measured across the 10,000 validation images.

## 3.1 Scene attributes & lane/object labels (bdd100k_labels.zip)

Every image JSON carries a top-level attributes block:

"attributes": { "weather": "clear", "scene": "highway", "timeofday": "daytime" }

Observed value ranges (val split):

weather: clear, overcast, partly cloudy, rainy, snowy, foggy, undefined

timeofday: daytime, night, dawn/dusk, undefined

scene (road type): city street, highway, residential, tunnel, parking lot, gas stations, undefined

Lane markings are labelled with type, color and style, for example:

lane/single white   -> {direction: parallel, style: dashed}
lane/double yellow  -> {direction: parallel, style: solid}
lane/road curb      -> {direction: parallel, style: solid}
lane/crosswalk      -> {direction: vertical, style: dashed}

Lane/area categories present: single/double × white/yellow/other, road curb, crosswalk, plus drivable area and alternative-drivable area.

## 3.2 GPS / IMU / timing telemetry (bdd100k_info.zip)

One JSON per clip provides actual capture time, location, speed and motion:

"startTime": 1503823480532,            // epoch ms -> 2017-08-27 (real capture date/time)
"gps": [{ "latitude": 32.5538, "longitude": 34.9573, "altitude": 79.4,
          "speed": 11.57, "timestamp": 1503823480999 }],   // 40 samples
"accelerometer": [ ...2047 samples ], "gyro": [ ...1826 samples ], "course": ...

Note: Despite this richness, BDD100K still has no pavement material, marking age, or maintenance-condition labels.

# 4. CULane — Geometry + Scene Difficulty Categories

Located at /shared/data/culane/. Annotations are lane point-lists; the only contextual metadata is a 9-way test-set categorization.

Annotation format (.lines.txt) — x y point pairs per lane line:

240.573 590 257.848 580 275.127 570 292.409 560 ...

train_gt.txt adds 4 binary lane-existence flags:

/driver_23_30frame/.../00000.jpg  /laneseg_label_w16/.../00000.png  1 1 1 1

The list/test_split/ folder buckets test frames into 9 conditions:

| Category | Frames |
| --- | --- |
| normal | 9,621 |
| crowd | 8,113 |
| night | 7,029 |
| no line | 4,067 |
| cross | 3,122 |
| shadow | 930 |
| arrow | 890 |
| highlight (dazzle) | 486 |
| curve | 422 |

These provide lighting/traffic context (incl. night) but no weather, marking type/color, or pavement information.

# 5. CurveLanes — Geometry Only, No Metadata

Located at /shared/data/Curvelanes/. Splits: 100,000 train / 20,000 valid / 4,527 test images.

Each label JSON has a single key, Lines (dense curve point lists):

{ "Lines": [ [ {"x":"521.32","y":"1439.0"}, {"x":"526.41","y":"1436.37"}, ... ], ... ] }

No weather, scene, time, marking-type, or pavement attributes of any kind. Its sole purpose is curved-lane geometry.

# 6. TUSimple — Geometry Only; Time Inferable from Filenames

Located at /shared/data/TUSimple/. Highway-only, daytime (per dataset readme). 3,626 clips of 20 frames each; only the 20th frame is labelled.

Label format:

{ "raw_file": "clips/0601/1494453497604532231/20.jpg",
  "h_samples": [160, 170, 180, ..., 710],
  "lanes": [[-2,-2,...,567,532,496,...], ...] }    // x per fixed y-row; -2 = no lane }

No weather / scene / marking / pavement labels. The only time signal is encoded in paths: label-file dates (label_data_0313, 0531, 0601 = MM/DD) and clip folder names, which are nanosecond epoch timestamps (e.g. 1494452385... -> 2017-05-10).

# 7. Conclusions for the Road Readiness Framework

If the framework requires pavement material, marking age/wear, or verified maintenance condition, none of these four datasets provide it — they are built for lane and object detection. The closest available proxies are:

Marking type / color / style → BDD100K only (white/yellow, single/double, solid/dashed).

Lighting / time / weather context → BDD100K (explicit weather/scene/time) and CULane (coarse night/shadow/dazzle buckets).

Geometry / curvature / visibility degradation → inferable from lane points in all four (e.g. CULane "no line", CurveLanes curvature) as an indirect signal of marking visibility — not a verified condition label.

In short, BDD100K is the only dataset with directly usable contextual metadata; the other three contribute lane geometry that must be paired with external sources to obtain pavement-condition information.